ENERGY AUDIT REPORT

Green City, Rahul Nagar, Shivane, Pune

House Energy Report

By Ritesh Jadhav

Table of contents

- Abstract………………………………………………………………………….03

- Objectives……………………………………………………………………….04

- Expected Outcomes………………………………………………………......04

- Methodology…………………………………………………………………....05

- Calculations…………………………………………………………………….06

- Results and Discussions………………………………………………………07

- Suggestions…………………………………………………………………….09

- Conclusion………………………………………………………………………10

ABSTRACT

An Energy Audit was conducted at Green City, Shivane, Pune in the state of Maharashtra. The major purpose of the energy audit was to help identify Energy

Management Opportunities (EMO) in a typical family dwelling. This was to give a full perspective of the contribution of family units to energy usage and consumption.

The study was carried out by listing all electrical appliances used in the house, noting their power ratings, estimating daily usage hours, and calculating their energy consumption in kilowatt-hours (kWh). The total estimated monthly energy consumption was then compared with the actual electricity bill to evaluate accuracy and identify variations.

The analysis revealed that major energy consumption was attributed to appliances such as the refrigerator, fans, and lighting systems. A difference was observed between calculated and actual energy consumption, which can be attributed to factors such as variation in usage patterns, standby power losses, and unaccounted minor devices.

Based on the findings, several energy-saving measures were suggested, including the use of energy-efficient appliances, reduction of standby power consumption, and optimization of appliance usage. The implementation of these measures can lead to a significant reduction in overall electricity consumption and cost.

This project highlights the importance of energy auditing as an effective tool for promoting energy efficiency and sustainable energy use in residential settings.

OBJECTIVES

- To analyse the pattern of electricity consumption in a residential household.

- To calculate the total energy consumption of various electrical appliances based on their power ratings and usage duration.

- To identify major energy-consuming appliances contributing to overall electricity usage.

- To compare the calculated energy consumption with the actual electricity bill and analyse the variation.

- To estimate potential energy savings by implementing the recommended measures.

- To promote awareness about energy conservation and efficient use of electrical appliances in daily life.

EXPECTED OUTCOMES

- A clear understanding of the pattern of electricity consumption in the household.

- Components such as Refrigerator, Chimney, Fans would consume comparatively higher energy

METHEDOLOGY

The energy audit of the residential household was conducted using a systematic approach to estimate electricity consumption and identify areas for energy conservation. The methodology adopted for this study is described as follows:

- Identification of Appliances
All electrical appliances used in the household were identified and listed, including lighting, fans, refrigerator, television, and other commonly used devices.

- Collection of Power Ratings
The power rating (in watts) of each appliance was noted from the nameplate specifications or obtained from standard reference values where direct information was not available.

- Estimation of Usage Pattern
The average daily usage time (in hours) and quantity of each appliance were estimated based on typical household usage patterns.

- Calculation of Energy Consumption
The daily energy consumption of each appliance was calculated using the relation:

Energy (kWh/day) = (Power × Time × Quantity) / 1000

Monthly energy consumption was then obtained by multiplying the daily consumption by 30 days.

- Preparation of Data Table
All collected and calculated data were organized into a tabular format using spreadsheet software for better clarity and analysis.

- Comparison with Actual Electricity Bill
The calculated total monthly energy consumption was compared with the actual electricity bill to evaluate accuracy and identify variations.

- Analysis of Results
The appliances consuming the highest amount of energy were identified, and patterns of energy usage were analysed.

- Identification of Energy Saving Opportunities
Based on the analysis, possible areas of energy wastage were identified and suitable energy-saving measures were suggested.

- Estimation of Energy Savings
The potential reduction in energy consumption was estimated by considering the implementation of suggested improvements such as efficient appliances and optimized usage.

The above methodology ensured a structured and logical evaluation of energy consumption in the household.

| Appliances | Power (Watt) | Hours/Day | Quantity | Energy (Wh/day) | Energy (KWh/day) | Energy (KWh/month) |
| --- | --- | --- | --- | --- | --- | --- |
| Led lights (Hall) | 10 | 5 | 2 | 100 | 0.1 | 3 |
| Led lights (Bedroom) | 10 | 1 | 4 | 40 | 0.04 | 1.2 |
| Led lights | 7 | 1 | 6 | 42 | 0.042 | 1.26 |
| Led lights (Kitchen) | 10 | 7 | 4 | 280 | 0.28 | 8.4 |
| Fan | 50 | 8 | 3 | 1200 | 1.2 | 36 |
| Refrigerator | 200 | 12 | 1 | 2400 | 2.4 | 72 |
| Router | 4 | 20 | 1 | 80 | 0.08 | 2.4 |
| TV | 40 | 6 | 1 | 240 | 0.24 | 7.2 |
| Chargers | 30 | 3 | 1 | 90 | 0.09 | 2.7 |
| Chimney | 200 | 2 | 1 | 400 | 0.4 | 12 |
| PC | 150 | 6 | 1 | 900 | 0.9 | 27 |
| Total |  |  |  |  | 5.772 | 173.16 |

CALCULATIONS: -

Formula:

Energy(kWh) = Power(W)×Time(hours)×Quantity​1000

For Refrigerator (important case):
Power = 200 W, Effective Time = 12 hours/day, Quantity = 1

Energy (KWh/day) =  200×12×11000

Energy (KWh/day) = 2.4 KWh/day

RESULTS AND DISSCUSSIONS: -

Figure: Monthly Energy Consumption Distribution

The pie chart represents the distribution of monthly energy consumption among various household appliances, with a detailed breakdown of lighting usage across different areas. It is observed that the refrigerator is the highest energy-consuming appliance, accounting for approximately 41% of the total energy consumption, followed by fans (21%) and the personal computer (PC) at 16%.

Lighting consumption is distributed across different sections of the house, with the kitchen contributing the highest among lighting loads (5%), followed by the hall (2%), while bedroom and small area lighting contribute relatively smaller shares. Other appliances such as the chimney (7%), television (4%), router (1%), and chargers (1%) have comparatively lower contributions.

This detailed representation helps in identifying not only the major energy-consuming appliances but also the specific areas within the household where energy optimization can be implemented effectively, particularly in high-usage zones like the kitchen and common areas.

Figure: Monthly Energy Consumption of Individual Appliances

The bar graph represents the monthly energy consumption (in kWh) of individual household appliances. It is observed that the refrigerator consumes the highest amount of energy, followed by fans and the personal computer (PC). These appliances contribute significantly to the overall electricity usage.

Other appliances such as lighting, chimney, and television consume moderate amounts of energy, while devices like routers and chargers contribute minimally.

The graph clearly highlights the appliances with the highest energy consumption in absolute terms, which helps in identifying priority areas for energy conservation and efficiency improvements.

ENERGY SAVING SUGGESTION

Based on the analysis of energy consumption, the following measures are recommended to reduce electricity usage and improve energy efficiency:

- Fans

- Replace conventional fans with energy-efficient BLDC fans

- Operate fans at optimal speed instead of maximum speed.

- Estimated saving: Up to 40–50% in fan energy consumption.

- Refrigerator

- Minimize frequent opening of the refrigerator door.

- Maintain optimal temperature settings.

- Ensure proper ventilation around the refrigerator.

- Defrost regularly if not frost-free.

- Estimated saving: 10–15% reduction in energy consumption.

- Personal Computer (PC)

- Use power-saving modes (sleep/hibernate) when not in use.

- Shut down the system when not required.

- Prefer laptops over desktops where possible.

- Estimated saving: 15–25% reduction in PC energy usage.

- Television and Entertainment Devices

- Turn off devices completely instead of leaving them on standby.

- Limit unnecessary usage time.

- Estimated saving: 5–10% reduction.

- Router and Chargers

- Switch off the router during night hours if not required.

- Avoid leaving chargers plugged in without use.

- Estimated saving: Small individually, but significant collectively over time.

- General Measures

- Turn off lights and appliances when not in use.

- Use energy-efficient (5-star rated) appliances.

- Conduct periodic energy audits to monitor usage.

- Spread awareness among household members regarding energy conservation.

By implementing the above measures, a significant reduction in total electricity consumption can be achieved, leading to lower electricity bills and more sustainable energy usage.

CONCLUSION: -

The energy audit of the residential household provided a clear understanding of electricity consumption patterns and the contribution of various appliances to total energy usage. The calculated monthly energy consumption was found to be lower than the actual electricity bill, which can be attributed to estimation errors, variation in appliance usage, and the presence of unaccounted loads such as standby devices and small electronics.

From the analysis, it was observed that the refrigerator is the highest energy-consuming appliance, followed by fans and the personal computer (PC). These appliances together account for a major portion of the total electricity consumption, as also indicated by the graphical representations.

Based on the findings, several energy conservation measures were suggested, including the use of energy-efficient appliances, optimization of usage patterns, and reduction of standby power consumption. Implementing these measures can lead to a significant reduction in overall energy consumption and electricity costs.

Overall, this study demonstrates that a simple energy audit can effectively identify areas of energy wastage and provide practical solutions for improving energy efficiency. It highlights the importance of conscious energy usage in daily life and encourages the adoption of sustainable practices at the household level.